Aim: Perform FFT and IFFT on discrete time signal.

PROGRAM:

import numpy as np

import matplotlib.pyplot as plt

# 1. Discrete time signal from lab slide (no external file needed)

signal = np.array([1, 2, 3, 4, 5, 6, 7, 8])

N = len(signal)

n = np.arange(N)

k = np.arange(N)

# 2. Compute FFT

fft_result = np.fft.fft(signal)

magnitude_spectrum = np.abs(fft_result)

phase_spectrum = np.angle(fft_result)

# 3. Compute IFFT

reconstructed_signal = np.fft.ifft(fft_result)

# 4. Plot outputs

plt.figure(figsize=(10, 8))

plt.subplot(4, 1, 1)

plt.stem(n, signal)

plt.title("Original Discrete Time Signal x[n]")

plt.grid(True)

plt.subplot(4, 1, 2)

plt.stem(k, magnitude_spectrum)

plt.title("Magnitude Spectrum |X[k]|")

plt.grid(True)

plt.subplot(4, 1, 3)

plt.stem(k, phase_spectrum)

plt.title("Phase Spectrum ∠X[k]")

plt.grid(True)

plt.subplot(4, 1, 4)

plt.stem(n, np.real(reconstructed_signal))

plt.title("Reconstructed Signal (IFFT)")

plt.grid(True)

plt.tight_layout()

plt.show()

CONCLUSION :

The Fast Fourier Transform (FFT) and Inverse Fast Fourier Transform (IFFT) were implemented on an 8-point discrete-time signal. By exploiting the symmetry and periodicity of the twiddle factors (WNk), the Cooley-Tukey decimation algorithm efficiently transformed the signal from the time domain to the frequency domain, reducing the arithmetic computational complexity from ON2 required by standard direct DFT to ONlog2N. The magnitude and phase spectra captured the spectral energy and phase shifts across discrete frequency bins. Computing the IFFT verified the reversibility of the transformation by restoring the original sequence values with zero reconstruction loss.